Grayline version 1.2

--------------------

What's new ! (Compared to version 1.1)

--------------------------------------

The program calculates the width of the grayline for your qth and for each

qth which is calculated. The width depends on the date and the latitude.

Optional correction for local time instead of UTC for easier reading.

For each calculated path the 'cq zone' is displayed. (Idea of Cesar PY2YP)

Updated and extended dxcc datafile. (Additional corrections by Cesar PY2YP)

Smoother scrolling.

Some minor bugs were fixed.

Main purpose of the program.

----------------------------

This program calculates possible grayline propagation paths for your location.

The sunrise/sunset algorithm is taken from 'Van Heddeghem', ON4HW and can be

found in the book 'Low-Band DXing' by John Devoldere ON4UN published by the

A.R.R.L. ( First edition Second printing page I-17 ). The width calculation

algorithm is from John Devoldere ON4UN published in the same book at page

I-6 and I-7.

System requirements : IBM PC with MS-DOS 5.0 or higher and 128 kB free RAM.

A mathematical coprocessor is not needed but will speed up the program.

The grayline.ini file should be edited by an ascii editor. The first line

is your latitude (north uses +, south uses -), the second line is your

longitude (west uses +, east uses -) and the third line the time offset

(hours) compared to UTC. Enter a zero if you want to display UTC times.

Corrections for Daylight Savings Time must be made manually !

The grayline.dat file is a comma delimited DXCC country list which can be

modified if there will be new dxcc entries.

The format is : prefix,city,country,latitude,longitude,cq-zone

prefix    : max.  5 positions

city      : max. 19 positions

country   : max. 19 positions

latitude  : decimal degrees (north uses +, south uses -)

longitude : decimal degrees (west uses +, east uses -)

cq-zone   : 2 digit integer between 01 and 40

The program is easy to use. It asks for what day you want grayline path

calculations and if you would like the results on your screen or written in

a text file. On the screen you can scroll through the results with the arrow

keys and switch between the predicted paths for sunrise and sunset with the

'PageUp' and 'PageDown' keys. Use the 'End' key to leave the program.

Credits to John Devoldere ON4UN and Van Heddeghem ON4HW.

Thanks to all the people who have sent positive critics about version 1.1

Any critics for improvement are still very welcome.

73, Folkert PA3CQR

E-mail : geurink@iri.tudelft.nl

WWW    : http://irian1.iri.tudelft.nl/users/pa3cqr/grayline.htm