RITTY 2.0

Copyright 1998 by Brian Beezley, K6STI

All Rights Reserved

RITTY 2.0......................................1

Evaluation Version.............................1

Use in Windows.................................1

Sound Card.....................................2

FSK, PTT, and TTY Output.......................3

Displays.......................................4

Using RITTY....................................5

Signal Processing..............................7

Setup Menu....................................11

Receive Options Menu..........................13

Transmit Options Menu.........................15

Baudot Punctuation............................17

Screen Colors.................................18

Implementation Notes..........................18

Use with RTTY by WF1B.........................19

Use with OH2GI and NA.........................23

Copyright and License.........................23

---- RITTY 2.0 -------------------------------------------------

RITTY combines a high-performance FSK modem with a

terminal program to enable you to transmit and receive

radioteletype and PACTOR signals on your PC.  The program uses

advanced digital signal processing algorithms but does not

require DSP hardware.  RITTY uses your sound card for analog

input/output and optimized assembly language for speed so it can

run right in your PC.  RITTY requires a 486 or better, math

coprocessor, VGA, sound card, and DOS 3.3 or later.

This document describes general aspects of RITTY and

RTTY mode.  See PACTOR.DOC for information about PACTOR mode.

---- EVALUATION VERSION ----------------------------------------

The unlicensed evaluation version runs for a limited

time and does not retain its state when restarted.  For full

functionality, put your RITTY.LIC license file in the directory

that contains RITTY.EXE.  (If you rename RITTY.EXE to XYZ.EXE,

rename RITTY.LIC to XYZ.LIC.)  The license file is valid only

for your callsign.  If you change it or will use a different

call for a contest, DXpedition, or special event, contact K6STI

for another license file.

---- USE IN WINDOWS --------------------------------------------

The following instructions assume that all RITTY files

are in \RITTY.

To install RITTY in Windows 3.x, select a program group

for RITTY in Program Manager.  Select File | New.  In the New

Program Object menu, select Program Item and click OK.  In the

Program Item Properties menu, under Description enter RITTY.

Under Command Line enter RITTY.  Under Working Directory enter

\RITTY.  Under Shortcut Key enter R.  Click Change Icon.  Click

OK when told that no icons are available.  In the Change Icon

menu, under File Name enter \RITTY\RITTY.ICO.  Click OK

repeatedly to dissolve all menus.

To install RITTY in Windows 95, start Explorer and

navigate to \RITTY.  Using the right mouse button, drag the

RITTY.EXE icon to the desktop.  Click on Create Shortcut(s)

Here.  Next, right-click on the dragged icon.  Click Properties,

then click the Program tab.  Click on Change icon.  Click

Browse, navigate to \RITTY, and double-click on RITTY.ICO.

Click OK twice.

To run RITTY, double-click on its icon.  Press Alt-Tab

to switch to another program.  To switch back to RITTY in

Windows 3.x, press Ctrl-Alt-R.  (Switching to another program in

Windows 3.x may disrupt copy).

1

When running RITTY in Windows, make sure background

software like a virus checker doesn't consume too much CPU time

and disrupt RITTY's signal timing.  This is particularly

important for PACTOR.

---- SOUND CARD ------------------------------------------------

RITTY works in RTTY mode with most sound cards, but

PACTOR requires a genuine, 16-bit Creative Labs card.  Other

cards may provide inadequate input/output levels, offer only

coarse gain adjustment, and exhibit limited dynamic range.

RITTY uses the SET BLASTER statement to determine sound-

card hardware settings.  You may need to configure your card to

these settings in AUTOEXEC.BAT by executing a utility like

DIAGNOSE.  For PACTOR the two DMA channels must differ.  See

your sound-card manual for details.  RITTY does not require

sound-card device drivers in DOS.

You can obtain receive audio from your transceiver's

speaker output, but it's better to use a fixed-level audio

output when available.  This lets you adjust AF gain for

comfortable listening without affecting RITTY.  Use slow AGC and

set RF gain to maximum.  Connect receive audio to the sound-card

left-or right-channel line input.  If your receiver can't fully

drive the line input, use the mic input instead.

NOTE:  It's easy to overdrive the sound card's high-gain

microphone preamp.  Reducing the input-gain setting won't help

because the preamp comes before the gain-control stage.

Therefore, it's important to check for distortion whenever you

use the mic input.  Tune your receiver past a strong, clean sine

wave.  Look for harmonics generated by an overdriven preamp in

the spectral display.  You should see just one signal pip as you

tune through the SSB passband.  If your receiver's fixed-level

output overdrives the mic input, add a series resistor (try 3.3k

ohms) to form a voltage divider with the 540-ohm mic input

resistance.

Adjust input gain in 2-dB steps using the Setup menu.

(Many sound cards have coarser steps and limited adjustment

range.  Some cards may provide no gain adjustment at all).  The

bar graph in the lower-center of the screen displays peak input

amplitude.  The graph turns red when the A/D converter clips.

Raise the input gain until clipping occurs, then back it off one

step.  Infrequent, brief clips won't affect copy so don't worry

if the graph occasionally flashes red.  However, sustained

overdrive will cause problems.

You can connect the mic, line-left, and line-right

inputs to separate signal sources.  Each input maintains its own

input-level setting so you can switch among the sources without

readjusting input level.  (Some sound cards may not provide

source selection.)

2

RITTY can provide AFSK or FSK output.  See the next

section for FSK.  RITTY generates AFSK on the left channel.

Route the left-channel line output (mini phone-plug tip) to your

transceiver's packet, phone-patch, or mic input.  VOX works fine

to switch between transmit and receive for RTTY, but PACTOR

requires push-to-talk (PTT) keying.  To use PTT, build the

interface circuit described in the next section.

To set AFSK transmit level, first disconnect your

microphone if active.  Switch to LSB and turn off your speech

processor.  Set your mic gain to the position you normally use

for voice (very important).  Set your power-output control to

maximum.  To use VOX, enable it, set anti-VOX and VOX delay to

minimum, and adjust VOX sensitivity so that your rig keys

reliably when you press Pause.

If you have a wattmeter, enter your transceiver's duty-

cycle rating for extended, key-down transmission in the Setup

menu, press Pause, and adjust Output Level in the Setup menu for

rated RTTY output.

With no wattmeter, temporarily enter 100% for rated duty

cycle.  Adjust Output Level to the point where your ALC just

begins to activate.  Then reset duty cycle to your transceiver's

rating.

Carefully check your transmit signal for hum and noise.

If present, add a 10k-ohm resistor in series with the line

output and a 330-ohm resistor across your transceiver's audio

input.  Then reset transmit audio level.

Once you've set Output Level by this procedure, it's OK

to use your mic-gain control to make minor power-output

adjustments.

RITTY generates phase-continuous, 16-bit, sine-wave

output (8-bit output when not using a 16-bit Creative Labs sound

card).  The signal is cleaner and more accurate than the FSK

provided by many transceivers.  (For example, the Kenwood TS-

930S and TS-940S generate 165.9-Hz FSK by filtering the square-

wave output of a digital divider and inserting this signal into

the SSB transmit audio chain.  This is AFSK in disguise!)

---- FSK, PTT, AND TTY OUTPUT ----------------------------------

Some transceivers won't let you use a CW filter in LSB

mode but allow it in FSK.  To permit use of a narrow IF filter,

RITTY can generate FSK output on a serial port (pin 2 for DB25,

pin 3 for DB9).  Select the port in the Setup menu.  Mark is a

negative output voltage and space is positive.  You may be able

to apply the RS-232 signal directly to your rig's FSK input.  If

not, use an interface circuit like that described next for the

PTT signal.  (To invert FSK polarity, swap the base and emitter

leads.)  Ground is pin 7 for DB25 and pin 5 for DB9.

3

RITTY drives both RTS and DTR (pins 4 and 20 for DB25,

pins 7 and 4 for DB9) of the selected serial port with a PTT

signal.  Transmit is a positive voltage, receive is negative.

You can obtain PTT from a parallel port instead (AFSK only).

RITTY drives all eight TTL outputs high for transmit and low for

receive.  For parallel-port output select COM port X in the

Setup menu.  For Port X Address enter 378 for LPT1, 278 for

LPT2, or 3BC for LPT3.  Connect a 1k-ohm resistor from the RS-

232 or TTL output pin to the base of an NPN transistor.  Ground

the emitter and connect the collector to your PTT line.

RITTY provides Baudot output on the serial port during

receive for hardcopy on a Teletype machine.  RITTY outputs at

most 72 characters per line, pads the data stream to provide

sufficient time for mechanical carriage return, inhibits

overprint, suppresses diddle for quieter operation, and ignores

stray LFs when enabled.  Engage your Teletype's unshift-on-space

mechanism to enable that feature.  RITTY disables TTY output

during transmit and when TSR.

---- DISPLAYS --------------------------------------------------

RITTY uses 640 x 480 graphics mode.  This allows 30

lines of text with custom fonts and special symbols.  It also

permits real-time signal display.

The lefthand graph is a spectral tuning indicator.  The

yellow trace is a fast Fourier transform (FFT) of the input

audio.  The cyan trace is the average spectrum.  Purple lines

show the channel-filter locations; line height is proportional

to filter output.  Tune your receiver to center the signal pips

on the purple lines.  Red arrows point to the right or left

whenever the strongest signal in the 3-kHz audio passband is

beyond the tuning-indicator range.  Red bars below baseline

indicate AFSK transmit frequencies.  Use the Setup menu to set

FSK center frequency, the up/down arrow keys to change frequency

shift (+/- also work), and the left/right arrow keys to fine-

tune the shift.

The righthand graph displays one of three waveforms.

The character waveform shows the demodulated mark-minus-space

signal for one character length.  The red tic marks locate

sample points for the start pulse, the five data bits, and the

stop pulse.  The D1 display is a sliding window of several

seconds of demodulated data and the detection threshold.  The

yellow dots are data samples.  Above the cyan trace a data point

is detected as mark and below as space.  D1 reveals the effects

of selective fading, static crashes, receiver AGC, Automatic

Threshold Correction, etc.  The D2 display is a stationary

version of D1.  D1 and D2 show the effects of Adaptive Channel

Combination; the character waveform does not.

4

---- USING RITTY -----------------------------------------------

Type RITTY at the DOS prompt to start the program.

Press any unused key to display a list of commands and

functions.  A menu appears when you press a command key.  Exit

the menu by pressing Esc.  You also can exit by pressing the

command key a second time.  This is handy when making a quick

check.  You can go directly from one menu to another by pressing

a command key.

To change a menu item, use the up/down arrow keys to

position the red pointer to the item.  If the item offers a

choice, use the left/right arrow keys to move the highlight to

another selection.  If the item is numerical, type in a new

value.  After typing all digits, press Esc to enter the item and

dissolve the menu.  The next time you bring up the menu, the

cursor will be where you left it.  Terminating numerical entry

with Esc is handy for repeatedly changing a single parameter to

observe its effect.  Press Enter instead of Esc to retain the

menu and advance the cursor to the next item.  Terminating entry

with Enter is good for changing several items at once.

Several menu items have hotkeys (underlined).  You can

alter one of these parameters without accessing a menu by

simultaneously pressing the Alt key and the hotkey.

Press Pause to enter transmit mode.  RITTY transmits

whatever you type.  Press Pause or Esc to return to receive.

Because Pause requires two keystrokes on some notebook

computers, RITTY also treats Print Screen as Pause.  (Use the

Windows 95 Properties menu to deselect Print Screen as a

shortcut key.  Use the Windows 3.x Advanced PIFEDIT menu to

select PrtSc as a reserved shortcut key.  These choices let

Windows pass Print Screen keystrokes to RITTY.)

You can transmit a canned message by pressing F1 through

F12.  Prepare these messages in receive mode.  (You can edit a

message while transmitting by pressing Alt-F1 to Alt-F12.)  Each

message can be up to 28 lines long.  Use any of the editing keys

listed, plus Backspace and Del.  Alt-W deletes a word, Alt-D

deletes a line, and Alt-C clears the entire message.  Press

^right-arrow or ^left-arrow to skip to the next or previous word

(^ means the Ctrl key).  Press ^R to enter an RX symbol.  When

RITTY encounters RX during transmit, it switches back to receive

mode.  You can also use ^R in transmit mode while typing ahead.

One message can invoke another using %.  For example:

F1   %9 DE K6STI

F2   HELLO %10

F9   N1RCT

F10  DICK

Pressing F1 yields N1RCT DE K6STI and F2, HELLO DICK.

Messages invoked by % can themselves invoke others.  A

concatenation limit of 2000 characters prevents infinite loops.

5

Use ^ (not a Baudot character) to refresh the shift

state.  This feature is useful in canned messages that contain

long strings without a shift change.  The LTRS or FIGS ^ becomes

can correct a shift hit taken at the receiver.  Alternatively,

you can have RITTY duplicate each LTRS/FIGS or periodically send

another.  See the Transmit Options Menu section for details.

(LTRS is the Baudot code that causes subsequent characters to be

interpreted as letters.  FIGS shifts the decoder to numbers and

punctuation.)

Use any of the message buffers to begin replying to a

station while he's sending.  If he turns it over to you while

you're still typing, don't bother to exit the message editor.

Simply press Pause.  The message will be copied to the transmit

buffer and you'll enter transmit mode.  You can finish typing

the message as it's being transmitted.

To transmit an ASCII file, enter the filespec (filename

preceded by directories, if necessary) in one of the message

buffers.  RITTY will transmit the file contents instead of the

filespec.  RITTY transmits nothing and echos a centered dot to

the screen for ASCII characters without Baudot equivalents.  %

has no significance in files, but ^ does.

Press Esc to erase the transmit buffer.  This is handy

after pressing the wrong function key.  Press Alt-M or Alt-S to

output a steady mark or space tone.  This is useful for

measuring transmit frequency or for checking the amplitude

ripple of your transmit-IF filter.

The only editing key in transmit mode is Backspace.  You

can backspace to erase any character on the current line not yet

transmitted (colored white).

RITTY can transmit characters immediately or it can

buffer them and transmit a complete word only after you've

followed it with space, Enter, or ^R.  Word output provides a

chance to correct a typo when you're not typing far ahead.

(After entering a space or ^R, backspacing reinhibits output to

provide yet another chance.)  Enable word output in the Transmit

Options menu.

Except when TSR, RITTY automatically records receive and

transmit text to a file named TEXT in the current directory.

After quitting RITTY you can edit TEXT to extract information.

RITTY overwrites TEXT each time it executes.  TEXT holds the

most recent 64 kB of data (about 3 hours of copy at 45 baud).

RITTY maintains its state between sessions (licensed

version only).  When you quit, RITTY saves all menu settings and

messages in RITTY.INI in the directory containing RITTY.EXE.  If

RITTY.INI exists when you start the program, RITTY reads the

file and reinitializes itself.  Otherwise it uses default

settings.  Delete RITTY.INI to return to factory defaults.

6

To start RITTY with a different set of parameters, you

can specify an initialization file other than RITTY.INI.  Put

the filespec (which must contain a period) anywhere on the

command line.  To create the initialization file, start RITTY

with a new filespec, set all parameters, and then quit.

---- SIGNAL PROCESSING -----------------------------------------

RITTY's purpose is to recover RTTY signals other modems

can't detect.  Here's how it works:

Input BPF

RITTY provides a 200-tap, linear-phase, FIR input

bandpass filter.  Use this filter for QRM when you lack a CW

filter or yours is too broad.  The BPF is 280 Hz wide at -6 dB,

425 Hz wide at -60 dB, has a double-humped response with a 16-dB

central depression (each hump is 100 Hz wide), and provides 54

dB of ultimate rejection.  Enable the filter with the Receive

Options menu.  It autotunes along with the channel filters.  The

BPF slightly degrades receive sensitivity when only noise is

present.

Above 45 baud RITTY uses the PACTOR BPF (500 Hz at -6

dB, 800 Hz at -60 dB, and ultimate rejection > 100 dB).

Although the BPF menu setting isn't reset, no filter is engaged

at 425- or 850-Hz shift.

Narrowband Detector

The narrowband detector maintains signal linearity as

long as possible.  It doesn't use an input limiter.  This allows

it to extract signals buried in noise and QRM without suffering

the limiter capture effect.

The narrowband detector uses a pair of matched filters,

one each for mark and space.  The filters have the SIN(x)/x

shape optimal for the detection of tone bursts in additive,

white, Gaussian noise.  The filter widths track baud rate and

the center frequencies are adjustable.

The matched filters feed an I-Q envelope detector.

RITTY does not do post-detection, low-pass filtering.  Any

filtering beyond that provided by the matched filters would

degrade sensitivity.

The matched-filter peaks are quite narrow at the slower

baud rates; tuning is critical.  Try to tune signals within 10

Hz.  Although the peaks are narrow, the filter skirts drop off

slowly.  Strong signals in the skirts may disrupt copy.  Use a

500-Hz CW filter or RITTY's BPF for heavy QRM.  (To use an

external DSP filter, see DSPFLTR.DOC.)  Even without a filter

7

the narrowband detector is less affected by QRM than ordinary

detectors because it doesn't use an input limiter.

ATC

RITTY provides a sophisticated Automatic Threshold

Correction algorithm.  ATC modifies the threshold that

distinguishes mark from space.  For tones of equal amplitude the

threshold is zero.  ATC alters the threshold to compensate for

amplitude imbalance due to selective fading, transmit or receive

IF-filter ripple, and audio roll-off.  ATC helps recover text

that otherwise would be garbled.  It lets you notch QRM in one

channel with your receiver's notch filter and maintain copy with

the remaining channel.  ATC peeks into the future to examine

upcoming signal characteristics to determine an optimal

detection threshold for the current character.  Display the D1

or D2 waveforms to view ATC action.

RITTY's ATC performs Adaptive Channel Combination prior

to threshold adjustment.  The mark and space channels of

constant-amplitude signals are combined with equal weighting.

But during selective fades the weaker channel may contribute

more noise than signal.  ACC adaptively adjusts channel

weighting coefficients to maximize S/N for the combined mark-

minus-space signal.  ACC can improve S/N up to 3 dB during deep

selective fades.

Wideband Detector

Although the narrowband detector maximizes sensitivity,

it does not respond well to signals corrupted by severe

ionospheric disturbance.  For example, polar flutter can

modulate a signal with AM and FM components.  Both components

can create incidental sidebands that broaden the signal spectrum

beyond the matched-filter passbands.  The FM component also can

degrade phase coherence.

RITTY provides a wideband detector for better copy of

corrupted signals.  The detector (a zero-crossing/extremum event

counter with a bit-length running window) has a very short

coherence time.  Signal phase can wildly fluctuate without

significantly affecting detector output.

The wideband detector implicitly contains a limiter and

therefore benefits from prefiltering.  RITTY normally engages

the BPF along with the wideband detector.  For slow computers

disable the BPF and use your CW filter instead.  The BPF or CW

filter improves detector sensitivity and lowers susceptibility

to QRM.  When you switch back to the narrowband detector RITTY

restores its BPF setting.  The wideband detector doesn't require

ATC; Autotune effectively provides this function.

When the wideband detector is active RITTY draws a

dotted line in the spectral display at the frequency that

8

corresponds to the event count that differentiates mark and

space.  The line height reflects the RMS signal amplitude at the

detector input.  Disable Autotune and use the line to map out

the BPF response (receiver AGC on) or to put RITTY's center

frequency in the middle of your CW-filter passband (AGC off).

Autotune

RITTY provides automatic BPF and channel-filter tuning.

When enabled, the filters quickly lock to the strongest tones

within the Autotune passband and then slowly track them.

Autotune accepts shifts between 135 and 250 Hz when using narrow

shift.  Press the spacebar to freeze the filter frequencies (the

filter markers turn gray).  Press again to unfreeze.  QRM may

capture the filters, but you can cycle through the three most

recent filter settings by pressing Tab.  Tab also freezes

Autotune.  Use the Receive Options menu to control Autotune

speed and passband.

You may need to freeze Autotune when running a large

pileup to prevent the channel filters from hopping among the

various tones.  The narrowband-detector filters are capable of

isolating one signal among many if you carefully tune.

Enable AFSK Autotune with the Transmit Options menu when

calling stations.  This automatically aligns the center

frequency of your transmit tones to that of the received signal.

Transmit shift will be 170 Hz for received shifts less than 176

Hz, 200 Hz for receive shifts greater than 191 Hz, and 182 Hz

otherwise.  (It will be 425 or 850 Hz for wide shifts.)

Autotune modifies the threshold count for the wideband

detector.  Effectively this provides ATC.  In addition, Autotune

facilitates reliable wide-detector squelch.

Numerical Flywheel

RITTY tries to lock a Numerical Flywheel to the incoming

data stream.  The flywheel synchronizes its rotation to the

received-character timing.  When locked, the flywheel provides

data-sample timing much less noisy than that derived from raw

start pulses.  The Numerical Flywheel can maintain lock on weak

signals and on strong signals through deep fades.  It provides

the precise synchronization necessary to avoid intersymbol

interference and to fully exploit the processing gain of the

matched filters.  Essentially the Numerical Flywheel

synchronously samples asynchronous RTTY.

The Numerical Flywheel is useless whenever characters

are sent at an irregular rate.  This can occur for hand typing

without diddle.  It also is characteristic of certain RTTY gear

even with diddle enabled.  For example, some firmware versions

of the Kantronics KAM TNC send most characters with 2 stop bits

but occasionally insert one with 2.5.  Vintage Tono terminal

9

units output diddle at half the normal character rate.  These

peculiar signals will unlock the Numerical Flywheel.  RITTY

detects irregular character emission and disables the flywheel,

but not before garbling a few characters.  It tries to relock

after any disturbance so the cycle will repeat.  Disable the

Numerical Flywheel with the Receive Options menu for signals

with irregular timing.

Better yet, select Auto.  In this mode the Numerical

Flywheel can unlock and relock without losing characters.

Although it won't remain locked as well in noise, Auto is best

for general use.  Use On to maximize text recovery for signals

with regular timing severely corrupted by noise, QRM, and QSB.

Evaluate Numerical Flywheel performance with the

character waveform display.  The sampling instants depicted by

the red tic marks should occur at or very near waveform peaks.

Unstable, off-peak tic marks may occur for signals with slightly

irregular timing that fail to disable the flywheel.  Manually

disable it to obtain good data samples for these signals.

When the Numerical Flywheel is locked RITTY displays

character length below the waveform display.  You'll see a wide

range of lengths transmitted on the air.  The standard length

for 45-baud RTTY with a 31-ms stop bit is 163 ms.  You'll

encounter signals with less than one stop bit (154 ms), to two

(176 ms), and even beyond.  The Numerical Flywheel can lock to

bit streams with 0.75 to 2.25 stop bits.

Repetition Integration

RITTY can detect text repetition even when incorrectly

decoded.  Repetition Integration (RI) aligns and integrates the

demodulated waveforms underlying repeated text so that signals

add coherently (S + S = 2S) but noise adds incoherently (N + N =

1.414N).  This enhances S/N by a factor of n for text repeated n

times.  RITTY highlights decoded integrated text brown (yellow

in WF1B).  Use RI to pull callsigns and other repeated text out

of the noise.  See the Receive Options menu for RI options.

Squelch

RITTY provides a sensitive squelch to inhibit print when

there's no signal.  You can adjust the squelch threshold or

disable it entirely using the Receive Options menu.  When the

squelch suppresses one or more characters, RITTY displays a red

dot.  A new character overwrites the dot when the squelch opens.

If red dots momentarily appear during a transmission, it means

you're missing text; lower the squelch threshold to recover

more.  Absolute input level does not affect squelch sensitivity,

but engaging a CW filter may.

10

---- SETUP MENU ------------------------------------------------

Source

Select the mic input only if your receiver can't

adequately drive a line input.  Be sure to check for sound-card

distortion when using the mic input.  See the Sound Card section

for details.  (Some sound cards may provide no input-source

selection.)

Text Font

This parameter determines the display font for receive,

transmit, and edit text.  Sans means sans serif.

Zero Style

RITTY provides plain, dotted, and slashed typefaces for

the digit zero.  Select the style you prefer.  Note that a plain

zero and the letter O have different shapes.

Audible Bell

Select Yes to have RITTY emit a short beep when it

receives a bell code.  Select No to display a cute little bell

symbol instead.

Input Level

Gain resolution is 2 dB for 16-bit Creative Labs sound

cards.  RITTY rounds down odd-numbered entries.  (Other sound

cards have coarser resolution and limited gain-adjustment range.

Older cards may provide no input-gain adjustment at all.)  Each

source maintains its own input-level setting.

Output Level

Gain resolution is 2 dB for 16-bit Creative Labs sound

cards.  RITTY rounds down odd-numbered entries.  (Other sound

cards have coarser resolution and limited gain-adjustment

range.)

Output

When you select FSK you must select a serial port for

output data.  The selected port also provides PTT for both FSK

and AFSK.  FSK output bits can't be software-inverted, FSK tones

can't track those of the received signal, and FSK transmit power

can't be dynamically adjusted.

11

COM Port

Select the serial port for FSK and PTT output.  Select

Unused for AFSK/VOX or X for a nonstandard port.

Aux Port

This auxiliary serial port provides timing for PACTOR

FSK.  It cannot be set to the serial port used for FSK output.

Select Unused for AFSK or Y for a nonstandard port.

Port X Address

Specify a nonstandard hex address for the COM port.  For

PTT output on a parallel port (AFSK only), enter 378 for LPT1,

278 for LPT2, or 3BC for LPT3.

Port Y Address

Specify a nonstandard hex address for the Aux port.

Port Y IRQ

Specify the IRQ for a nonstandard Aux port.

Rated TX Duty Cycle

Enter your transceiver's percentage-of-full-output

rating for extended key-down transmissions.  For example, if

your 100-watt transceiver is rated at 50 watts output for RTTY,

enter 50%.  RITTY then will automatically adjust AFSK output to

the proper level for all transmit modes.

Center Freq

Center frequency is halfway between the mark and space

tones.  When using a CW filter, adjust this parameter to center

the tones in the filter passband.  Otherwise you might consider

using low-frequency tones that are more pleasant to listen to.

RTTY Baud

You can select one of the following transmission speeds:

Speed           Element Length

45 baud              22 ms

50                   20

57                   17.6

12

75                   13-1/3

100                   10

---- RECEIVE OPTIONS MENU --------------------------------------

Flywheel

Select Auto for general use.  Use Off for the rare

signal with timing too irregular for Auto to handle.  Select On

for best copy of regular signals through slow, deep fades or

those buried in noise.

Detector

Select Narrow for normal use.  Use Wide for signals

corrupted by polar flutter.  The narrowband detector maximizes

sensitivity.

Wide Det BPF

This parameter determines the BPF state for the wideband

detector.  Select Off and use your CW filter when your computer

is too slow to run the BPF.

BPF

Enable the input bandpass filter for heavy QRM.  The

filter autotunes along with the channel filters.  It

automatically disables itself at 425- or 850-Hz shift.  The BPF

requires much computation and may not run on slower computers.

ATC

For normal reception select On to engage Automatic

Threshold Correction and Adaptive Channel Combination.  Select

Off to observe D1 and D2 waveforms undistorted by ACC.

Autotune

Select S, M, or F for slow, medium, or fast Autotune

response speed.  Press the spacebar to freeze the filter

frequencies.  Press Tab to cycle through the three most recent

filter settings.

Autotune Width

Use this parameter to narrow the width of the Autotune

window to help keep QRM from capturing the filters.

13

Squelch Threshold

The squelch threshold adjusts in 1-dB steps.  0 disables

the squelch.  Use this setting for very weak signals, those with

deep QSB, or whenever you want to be sure to miss nothing.

Unshift on Space

Unshift-on-space reverts the decoder to LTRS mode

whenever a space is received.  This helps prevent printing long

strings of wrong-case text.  However, some modems don't

anticipate use of this feature and will print incorrectly.

Disable unshift-on-space for these signals.

On transmit, RITTY assumes that unshift-on-space may be

used at the receiver.  Therefore, in FIGS mode it always sends

LTRS or FIGS before the first nonspace character following a

space.  This procedure provides compatibility with all decoders.

Ignore Stray LF

RITTY can ignore LF codes that don't occur in the first

print column.  This helps keep more text on your screen when

copying noisy signals.  LF is seldom (if ever) transmitted

except in the first column.  (LF stands for line feed.)

Decode Both Shifts

RITTY can simultaneously print an alternate character

stream that uses the other LTRS/FIGS sense.  This feature is

handy when DXing weak and fading signals that may get stuck in

the wrong shift.  You'll find the correct shift sense on one of

the two lines displayed.  Often this will give you the crucial,

missing part of an incomplete callsign.

When you're not displaying both shifts, you can recover

from shift errors by looking northwest from the QWERTY...

keyboard row to identify corresponding digits.

Show Control Codes

RITTY can print symbols for the CR, LF, LTRS, FIGS, and

BLANK control codes.  This can reveal interesting quirks and

properties of various modems.  It also can show the redundancy

employed by some stations to combat noise (for example, W1AW

bulletins).  (CR stands for carriage return.  BLANK is a

nonfunctional code.)

14

Display Case

You can display received text in upper or lower case.

Upper case is always used when TSR.

Rep Integ

Select All to enable Repetition Integration for all

text.  Select Digit to require that repeated text contains at

least one digit.  This suppresses most of the false repetitions

that All often detects.

Display RI

Select Differ to display Repetition Integration only

when the individual repetitions differ.  Select Always to

display RI whenever it detects repetition.  Use Differ to reduce

screen clutter and Always to evaluate RI.

Waveform

Select Character for a high-resolution, character-length

display of the demodulated mark-minus-space signal.  Select D1

or D2 to view the signal envelope and ATC action.

---- TRANSMIT OPTIONS MENU -------------------------------------

Word Output

Select Yes to enable whole-word output.  Word Output is

disabled when TSR.

Duplicate LTRS FIGS

RITTY can add transmit redundancy by sending each LTRS

and FIGS twice.  This is helpful when your signal is weak or

you're trying to bust a pileup.  However, it slows transmission.

Refresh LTRS FIGS

RITTY can automatically resend LTRS/FIGS after a certain

number of characters has elapsed.  Enter the number, or 0 to

disable.  This feature provides an alternative to duplicating

each LTRS/FIGS and to using ^.

Add Begin/End CR

RITTY can automatically add a CR when you go to transmit

mode and another when you return to receive.  This helps ensure

15

that your transmit text isn't printed next to noise characters.

(RITTY doesn't add a CR when you interrupt output with Esc or

Pause, nor when one's just been sent.)

AFSK Autotune

Select On to automatically set AFSK center frequency to

that of the received signal.  Transmit shift will be 170, 182,

200, 425, or 850 Hz based on received shift.  AFSK Autotune is

useful when calling stations.  Turn it off when calling CQ so

your transmit signal stays put.  AFSK Autotune works only when

receive Autotune is enabled.

Fixed Shift

This is the transmit frequency shift when AFSK Autotune

is disabled.  Normally you won't need to set this parameter

because it tracks changes you make to receive shift with the up/

down arrow keys.  (Transmit shift doesn't track changes made

with the left/right arrow keys.  This lets you fine-tune a

signal and still reply at the correct shift.)  If you manually

set transmit shift, don't forget that the value will change

whenever you alter receive shift with the up/down arrow keys.

While narrow shift today is standard for amateur RTTY,

wide shift plus ATC provides better immunity to signal fading.

See WIDE.DOC for details.

Stop Bits

You can select 1, 1.4, 1.5, or 2 AFSK stop bits.  1.4

really is 31/22 stop bits (31 ms at 45 baud), the standard for

Teletype machines at 45 and 57 baud.  Most amateur RTTY signals

still use this length today.  1.5 stop bits is common at the

other baud rates.  Some TNCs transmit 2 stop bits.  This slows

transmission and widens the receive window during which noise

can generate a false start pulse.  Nevertheless, RITTY provides

this length for testing or other special purposes.  Finally, you

can select 1 stop bit.  This length sometimes is used at the

higher baud rates and can noticeably speed transmission at 45

baud.  However, some decoders won't reliably synchronize to

signals with stop bits this short.

Whenever you change baud rate, RITTY resets the transmit

stop-bit length to 1.4 at 45 and 57 baud and to 1.5 at 50, 75,

and 100 baud.  If you manually set the number of transmit stop

bits, don't forget that the setting may change when you alter

baud rate.

The serial-port UART determines FSK stop-bit length.  It

is always 1.5 (or very close).

16

New Line

RITTY provides four choices for the character sequence

emitted when you press Enter or when RTTY by WF1B sends CR.  CR

is the shortest possible sequence; use it during contests when

you're determined to minimize overhead.  The default CR LTRS

sequence refreshes the shift state at the beginning of each

line.  (RITTY sends FIGS instead of LTRS when the first

character of a new line requires it.)  Both of these sequences

work fine when decoded by terminal programs that won't overprint

lines.  CR LF LTRS works even for primitive terminal programs

that don't inhibit overprint.  Finally, use CR CR LF LTRS LTRS

when transmitting to a Teletype machine.  This sequence provides

time for the carriage to return to the first print column.

(However, it may cause some terminal programs to erroneously

double-space lines.)  RITTY limits the transmit line length to

72 characters when you select this sequence.

RITTY appends LFs to consecutive CRs when New Line is CR

or CR LTRS to enable blank lines to be reproduced by decoders

that properly ignore consecutive CRs.

Tones

This parameter controls tone inversion for both transmit

and receive.  (Sharing the parameter prevents inadvertent

reverse transmission.)  The amateur FSK standard is that the RF

mark frequency is higher than that for space.  Nearly all

commercial RTTY signals use the opposite convention.  Selecting

Reverse is more convenient than switching to USB and retuning.

---- BAUDOT PUNCTUATION ----------------------------------------

Unfortunately, punctuation for the Baudot code isn't

completely standardized.  RITTY provides two punctuation sets.

The first is used on Teletype Corporation printers.  The second

set is from the International Telegraph Alphabet No. 2.  RITTY

represents the unassigned FIGS-D, FIGS-F, FIGS-G, and FIGS-H

ITA2 codes by .  These codes may have country-specific meaning.

You'll find both punctuation sets in use on the air today.  To

change sets, press P and then up/down arrow.  The selected

punctuation set also is active on transmit.

To modify a punctuation mark, select it with the left/

right arrow keys and then type your replacement.  Use ^G to

enter the bell character and Alt-127 to reenter .  You can

enter extended-ASCII characters up to Alt-191.

If while transmitting you press a key not defined in the

current Baudot character set, RITTY emits a beep to alert you

that the character can't be transmitted.  (Exception:  RITTY

sends '' when " is undefined.)

17

You can create a composite punctuation set by

substituting ' for bell in the Teletype set.  This code will

print all signals on the air intelligibly.  (RITTY will transmit

FIGS-J for '.)

---- SCREEN COLORS ---------------------------------------------

Press C to set screen colors.  (To set them all, enable

Show Control Codes in the Receive Options menu.  Then enter some

transmit text and press Esc before it's all sent.)  Select a

screen item with the left/right arrow keys (an item blinks once

when selected).  Then use the Home, End, up/down arrows, PgUp,

and PgDn keys to cycle through the 64 intensities available for

each of the red, green, and blue color components.  Press Esc to

exit the Colors menu.

---- IMPLEMENTATION NOTES --------------------------------------

RITTY ignores the Baudot BLANK character on receive

(except when showing control codes) and never transmits it.

RITTY never overprints a line; when it receives CR

without LF, it advances print to the next line.  When it

receives consecutive CRs, it doesn't erroneously multispace

lines.  RITTY displays all characters received; when it receives

more than 80 on one line, it wraps print to the next.

Whenever you enter more than 80 characters on a transmit

line, RITTY wraps the display to the next line but doesn't

transmit any control codes.  Exception:  When the new-line

sequence is CR CR LF LTRS LTRS, RITTY wraps text after 72

characters and transmits the new-line sequence.

RITTY emits three spaces when you press Tab.  This isn't

true tabulation but it does provide a convenient way to indent

paragraphs.

When you switch to transmit, RITTY first sends LTRS for

synchronization and then CR, if programmed.  Finally, it sends

LTRS or FIGS before the first text character.  When you return

to receive, RITTY adds only CR, if programmed.

Except when generating pure tones with Alt-M or Alt-S,

RITTY outputs LTRS diddle when it has nothing to send.  See

DIDDLE.DOC for the reason.

RITTY programs the sound card for approximately 6000

samples per second.  This allows a maximum input or output

frequency of 3000 Hz.  Creative Labs sound cards have built-in

antialias filters.  There is some input roll-off above 2500 Hz,

but with ATC this has no effect on received signals, even for

850-Hz shift at a center frequency of 2210 Hz.

18

I measured the following output response for my Sound

Blaster 16:  -3 dB at 2650 Hz, -6 dB at 2710 Hz, and more than

12 dB down above 2800 Hz.  If you transmit 850-Hz shift at a

center frequency of 2210 Hz, the space tone will be 3 dB weaker

than mark.  You may want to lower the center frequency to

equalize tone levels for this particular case.  For narrower

shifts your IF filter likely will cause more roll-off than the

sound-card antialias filter.

---- USE WITH RTTY BY WF1B -------------------------------------

RITTY can act as a modem for WF1B's RTTY contest-logging

program.  To use it this way, type RITTY CONTEST.INI to create a

new initialization file.  Check all menu settings.  Those

recommended for contest use are listed below.  Exit RITTY and

type RITTY I CONTEST.INI to install the program in memory using

the contest settings.  Then start WF1B.  Select the K6STI TNC in

the work sheet.  The tuning indicator is a simplified version of

RITTY's FFT display.  The signal pips are within the marker bars

for a properly tuned signal.  After exiting WF1B, type RITTY U

to uninstall RITTY from memory.

You can automate this procedure with a little batch

file.  See CONTEST.BAT for an example.  (CONTEST.BAT invokes the

FONT.EXE utility to provide enhanced WF1B screen fonts.  Type

FONT for more information.)  RITTY sets ERRORLEVEL to 1 when an

error occurs so you can take evasive action in a batch file.

If you can't load RITTY and WF1B simultaneously, try to

make upper-memory blocks available.  RITTY can use up to 23K of

upper memory to free space for WF1B (DOS 5 or later).  If you

have lots of upper memory, try the LH RITTY command to load

RITTY entirely there.  See your DOS manual for memory-management

information.

RITTY works best with the real-mode version of WF1B

(www.wf1b.com/files/rttyreal.zip).  This version may not provide

enough memory for MASTER.CAL or FRIEND.INI.

I recommend the following parameter settings for

contests.  Settings marked -- don't matter.

Receive Shift           182 Hz

Punctuation             TTY with ' replacing bell

Colors                  --

Setup

Source                  As required

Text Font               --

Zero Style              --

Audible Bell            --

Input Level             As required

19

Output Level            As required

Output                  AFSK (if you can use your CW filter)

COM Port                As required

Aux Port                --

Port X Address          As required

Port Y Address          --

Port Y IRQ              --

Rated TX Duty Cycle     100%

Center Freq             Center of your CW-filter passband

RTTY Baud               45

Receive Options

Flywheel                Auto

Detector                Narrow

Wide Det BPF            On (Off if your CPU is too slow)

BPF                     On (Off if your CPU is too slow)

ATC                     On

Autotune                M

Autotune Width          500 Hz

Squelch Threshold       0 or 1

Unshift on Space        Depends on contest exchange

Ignore Stray LF         Yes

Decode Both Shifts      --

Show Control Codes      --

Display Case            -- (upper when TSR)

Rep Integ               Digit

Display RI              Differ

Waveform                --

Transmit Options

Word Output             -- (disabled when TSR)

Duplicate LTRS FIGS     No

Refresh LTRS FIGS       0

Add Begin/End CR        Yes (unless you add them in WF1B)

AFSK Autotune           -- (control with Alt-T & Alt-F in WF1B)

Fixed Shift             182 (set only after you set RX shift)

Stop Bits               1.4

New Line                CR LTRS

Tones                   Normal

Receiver AGCs often allow peak amplitudes for static

crashes to exceed normal signal levels.  Set the input level to

avoid frequent clipping on the noisiest band you plan to operate

during the contest.

Be sure to set RITTY's center frequency to the center of

your CW-filter passband.  Typical CW filters have poor group-

delay characteristics near their passband edges.  These filters

can seriously distort FSK signals even when their amplitude

response is flat.  Target the filter center to keep signals as

far from the passband edges as possible.  If you can choose

20

between 250-Hz and 500-Hz filters, use the latter unless you've

carefully evaluated the narrow filter's time-domain FSK response

with the waveform display.

The recommended fixed transmit and receive shifts better

accomodate the 200-Hz shift used by unmodified TNCs.  Spacing

filters 182 Hz does not compromise reception of 170-Hz signals

because it reduces interchannel crosstalk.  (182 Hz is within

0.2 Hz of the shift that nulls 45-baud crosstalk for matched

filters.)  If you're a shift bigot and insist on using 170 Hz,

you'll degrade receive sensitivity about 2 dB for 200-Hz

signals.  Degradation is less than 1 dB at 182 Hz.  Better

still, use Autotune.

Press Alt-T in WF1B to cycle through Autotune off, RX

Autotune, and RX plus TX Autotune (the last state appears for

AFSK only).  RITTY adjusts the tuning-indicator scale so that

autotuned signal pips line up with the markers even for

nonstandard shifts.  Disable TX Autotune when calling CQ so your

transmit frequency doesn't wander.  Press Alt-F to freeze

Autotune (press again to unfreeze).  Freezing Autotune is

particularly useful when running a large pileup.  If other

callers capture the channel filters when searching and pouncing,

try S Autotune speed.  For FSK disable Autotune when searching

and pouncing to ensure that your transmit tones are on

frequency.

The Autotune window is 500 Hz wide by default.  You can

decrease its width so that QRM is less likely to capture the

filters (but you'll have to manually tune some callers).  If you

make the window something like 250 Hz, Autotune acts more like

automatic fine tuning.

A threshold setting of 0 disables the squelch and

ensures that no signals are suppressed, however weak.  But

RITTY's squelch algorithm is very sensitive.  A setting of 1

invokes a minimal threshold that inhibits most noise characters

but passes extremely weak signals.  Using squelch makes it

easier to comprehend the screen at a glance.

Press Alt-~ in WF1B to toggle between narrow and wide

detectors.  WF1B displays "Polar" when the wideband detector is

active.  (~ is intended to suggest a wavy signal).  It's a good

idea to start WF1B with the narrowband detector and its BPF

setting.  RITTY then will restore this setting whenever you

toggle back to narrow with Alt-~.  (If your keyboard doesn't

have ~, use Alt with the key just below Esc.)

Engage RITTY's BPF along with a 500-Hz CW filter if you

can.  The IF filter will let you hear weak callers next to

strong.  The double-humped BPF shape will help RITTY recover a

signal even when interlaced among the tones of many others.  CW

filters narrower than 500 Hz restrict Autotune range and their

nonflat group delay may cause intersymbol interference.

21

Both the Teletype and ITA2 punctuation sets are adequate

for contest use.  The recommendation above makes operating more

enjoyable by silencing annoying bell codes generated by noise.

But here's an interesting idea from N1RCT:  Convert punctuation

not normally used during a contest to its corresponding LTRS

code.  If that code wasn't sent, it's invalid anyway.  You must

decide which characters to convert; for example, you'll never

need an apostrophe for a contest exchange but you might want one

while chatting with a buddy during a break in the action.

Some modems don't anticipate the use of unshift-on-space

on receive.  After sending a number and a space, these modems

don't resend FIGS before sending another number.  Therefore,

contest exchanges with more than one number (or numerical

repeats) may become garbled.  If the only number in the exchange

is RST, it's probably best to enable unshift-on-space.  (Just

remember that TOO means 599).  If the exchange involves

additional numbers, you may want to disable the feature.  You

can always use Alt-L in WF1B to force the shift state back to

LTRS (after the fact).  Better still, when expecting a number

WF1B will convert the letter-equivalent.  Just type or click on

what's received.

You can select All for Rep Integ to detect all repeated

text, not just that containing a digit, but doing so will

generate numerous false repeats.  Digit greatly reduces falsing

and is best for most contests.  It will detect repeated

callsigns and numerical contest exchanges.  Use Differ instead

of Always for Display RI to reduce screen clutter by displaying

RI text only when individual repeats differ.

You can make your transmit signal more robust in noise

and QRM by sending extra LTRS/FIGS.  RITTY can automatically

duplicate these codes or it can periodically resend them.

However, the most efficient way to add redundancy is to put ^

exactly where you want the shift state refreshed in a canned

message.  For example, a good place might be just before an

exchange repetition.

RITTY does not echo to WF1B the CRs it adds to the

beginning and end of a message.  This conserves screen space.

Make certain you do not set the number of transmit stop

bits to 1 or 2 for a contest.  Some TNCs won't reliably copy

signals with 1 stop bit no matter how loud they are.  Using 2

slows your transmission speed and makes your signal more

vulnerable to false start pulses caused by noise.

The only multiline messages you're likely to transmit

during a contest are long CQs.  Your callsign suffix resets the

shift state to LTRS just prior to each new line.  However, if

the LTRS is missed, everything up to your callsign suffix on the

next line will be interpreted as FIGS (without unshift-on-

space).  Using CR LTRS for the new-line sequence is cheap

insurance.  Anything longer is overkill.

22

---- USE WITH OH2GI AND NA -------------------------------------

To use RITTY with the OH2GI Ham System or K8CC's NA

contest logger, follow the WF1B procedure but install using

RITTY X instead of RITTY I.  See the NA manual for control

information.  Use the following keys to control RITTY in OH2GI:

Alt-R   RX Autotune

Alt-A   TX Autotune

Alt-D   Freeze Autotune

Alt-W   Wideband Detector

---- COPYRIGHT AND LICENSE -------------------------------------

This software is copyrighted.  It's licensed, not sold.

When you purchase a license for a software product, you're

granted the right to use the software under certain conditions;

ownership of the software remains with the copyright holder.

Licensing software is different than purchasing a tangible

object that you may use or dispose of as you wish.

This software is licensed to individuals at discount for

amateur use.  It has been provided to you on the condition that

you abide by the following:

1.  You will not use the software for professional, business,

government, military, or institutional purposes.

2.  You will not sell, rent, lend, give away, or otherwise

transfer your RITTY.LIC license file to others.

Brian Beezley, K6STI       (760) 599-4962

3532 Linda Vista Dr.        0700-1800 PT

San Marcos, CA 92069        k6sti@n2.net

23